| FINAL REPORT TO: |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Department of Transportation |  |  |  |  |  |  |  |  |  |
| National Highway Traffic Safety Administration |  |  |  |  |  |  |  |  |  |
| Office of Acquisition Management (NPO-320) |  |  |  |  |  |  |  |  |  |
| West Building 51-117 1200 New Jersey Avenue, SE Washington, DC 20590 |  |  |  |  |  |  |  |  |  |
| for |  |  |  |  |  |  |  |  |  |
| Contract Number: DTNH2216D00037/0002 |  |  |  |  |  |  |  |  |  |
| Cost and Weight Analysis of Blind Spot Detection Systems |  |  |  |  |  |  |  |  |  |
| from: |  |  |  |  |  |  |  |  |  |
| Ricardo Inc. |  |  |  |  |  |  |  |  |  |
| Detroit Technical Center |  |  |  |  |  |  |  |  |  |
| Van Buren Twp., MI |  |  |  |  |  |  |  |  |  |
| 48111 USA |  |  |  |  |  |  |  |  |  |
| 11 July 2018 |  |  |  |  |  |  |  |  |  |
| Approved: |  | Mark Kuhn |  |  |  |  |  |  |  |
|  |  | Vice President |  |  |  |  |  |  |  |
| Commercial Contact: |  | Melissa Poe |  |  |  | Tel: |  | 734-394-3798 |  |
|  |  | Finance Administrator |  |  |  | Fax: |  | 734-397-6677 |  |
|  |  |  |  |  |  | Email: |  | Melissa.Poe@Ricardo.com |  |

Table of Contents

1.0 ABSTRACT	2

2.0 SUMMARY OF FINDINGS	2

3.0 ENGINEERING ANALYSIS	5

Technology	5

Selection of Vehicles	6

BSD System Descriptions for Selected Vehicles	8

Component Procurement and Disassembly	12

Determination of Applicable Components	13

Cost and Weight Analysis Approach	14

Costing Approach and Assumptions	15

4.0 COST AND WEIGHT ANALYSIS	17

Cost and Weight Results	17

Kia Sedona	18

Ford F-150	21

Chrysler Pacifica	24

Comparison of Evaluated BSD Systems and Summary	27

5.0 ACRONYMS	30

6.0 APPENDICES	30

BOM for Kia Sedona	30

Cost and Weight Summary for Kia Sedona	33

BOM for Ford F-150	34

Cost and Weight Summary for Ford F-150	39

BOM for Chrysler Pacifica	40

Cost and Weight Summary for Chrysler Pacifica	43

# 1.0 ABSTRACT

The National Highway Traffic Safety Administration (NHTSA) recognizes the safety benefits of driver alert systems and is engaged in considerable research related to evaluating the effectiveness of alterative collision warning interface designs and implementation, as well as procedures for gauging such effectiveness, including impacts on vehicle costs and weight.  The purpose of this study was to establish reliable cost and weight estimates for Blind Spot Detection (BSD) Systems associated with passenger vehicles.

BSD systems use sensors to detect vehicles in adjacent lanes that may not be directly observed by the driver. The BSD alerts the driver that another vehicle may be present and to use caution if planning a lane change.  The detection zones include lateral and rear zones. If any vehicle arrives to the predefined detection zone, a warning signal is issued. In contrast, a warning might not be followed, depending on the BSD system, when the other vehicle is in the rear zone.  In detecting vehicles, the most frequent types are optical or radar. The sensor detects closing vehicle speed and its relative distance. Typical components associated with the BSD systems include the sensor (radar, optical), warning system (audible, haptic, side mirror indication), control unit, and associated wiring.

Ricardo Strategic Consulting (RSC) and NHTSA have selected and studied the BSD cost and weight impact on three light passenger vehicles: the Kia Sedona, Ford F-150, and Chrysler Pacifica.  The study included dismantling of the BSD systems and evaluation of components to determine manufacturing methods, weights, and costs. For the three vehicles studied, it was found that the average increase in weight was 0.89 kg and the average manufacturing cost increase was $112.13 and the average retail price increase to the end user for these systems was determined to be $145.93.

# 2.0 SUMMARY OF FINDINGS

Ricardo worked with NHTSA to define the study scope, baseline equipment, study assumptions, and cost methodology for light duty vehicles. The process involved three key procedures: 1) vehicle research, 2) vehicle segregation, and 3) vehicle selection.  The Kia Sedona, Ford F-150 and Chrysler Pacifica were selected for teardown analysis, which includes cost and weight determination, for each BSD system component.  The average annual sales for these vehicles for the years 2015-2017 were: Kia Sedona 34,945; Ford F-150 832,619; and Chrysler Pacifica 89,900.

Typical BSD components include:

- Sensors (typically radar, located near tail light assembly or integrated with this assembly)

- Control units (PCBs for sensors – antenna, LED control)

- Associated wiring harnesses

- Mounting hardware specific to BSD system

- Other materials and scrap (for electronic parts, this category includes  labels, soldering materials, flux, and fasteners)

Owner’s manuals, service manuals, OEM parts descriptions, and interviews with service technicians were employed to identify subassemblies, components, and parts associated with the BSD systems.  Detailed evaluations, including identification of sensors, displays, PCB pin assemblies, associated wire systems, and mounting hardware, ensured that only those items related to the BSD system were included in the cost and weight determinations.  The identified parts were then purchased through local dealers.

Ricardo engaged automotive system and vehicle integration experts, cost modeling teams, and procurement professionals to support the cost and weight analysis.  Following acquisition of the parts from dealers, analyses were performed to determine which subassemblies, components, and parts were uniquely associated with the BSD system.  The parts were then disassembled (“tear-down” procedure), catalogued, evaluated to determine manufacturing process, evaluated to determine materials used, weighed, and analyzed using specialized manufacturing cost software.

The chart below shows comparisons of the study results for the three BSD Systems.  Total manufacturing costs for BSD systems range from $92.70 to $129.27, with an average of $112.13.

The average manufacturing costs are further characterized by four elemental cost components of labor, material, variable and fixed costs, as described in Section 3.0. These manufacturing cost components are shown in the table below.  Overhead burdens, characterized by fixed percentage markups for SG&A, profit, transportation & warranty; as well as dealer costs and markup, are applied to the total manufacturing costs to determine the end-user cost increase of $145.93 for the average of the 3 vehicles studied.

The added weights of the three BSD systems are summarized in following chart.  They range from 0.43 kg to 1.35 kg, with an average of 0.89 kg.

.

# 3.0 ENGINEERING ANALYSIS

## Technology

BSD systems are in a category of devices termed advanced driver assist systems (ADAS), a class of mechanical and electronic features installed to enhance drivability and safety.

BSD sensors fall into two primary categories: radar and computer vision. These systems employ sensors to detect the presence of other vehicles in the equipped vehicle’s left and right blind zones (see Figure 1). The BSD processes the sensor information and presents it to the driver in the form of visual and audible indicators. Visual indicators are often integrated in the glass of both left and right outside rear-view mirrors, or inside the vehicle on the “A-pillar” or window frame near each outside mirror. Radar-based BSD systems for passenger cars use sensors that are typically mounted in left rear and right rear positions on a vehicle, either behind the rear bumper fascia or behind each rear quarter panel. The sensors transmit and receive radio waves to and from the vehicle’s left and right blind zones. The systems process the radar signals and provide visual and/or audible information to the driver. Snow, rain, and fog can disrupt the radar signals. Dirt, snow, and ice present on the surfaces through which the radio waves are transmitted and objects mounted on the vehicle near the sensors, such as bicycle and luggage carriers, can cause interference.

Computer-vision-based BSD systems use digital camera imaging technology to sense the presence of vehicles in blind zones. Cameras mounted on or near the outside rear-view mirror housings on both sides of the vehicle provide views of the blind zones to the system. Images from the cameras are processed by the systems using algorithms that identify image characteristics associated with vehicles. Systems may identify vehicle shapes in daylight, vehicle headlights at night, and/or some combination of shapes and lighting. The systems can be disrupted by snowfall and rainfall, and by snow, ice, and dirt present on the camera lenses. Shadows, light reflected from wet roadways, and sunlight at sunrise and sunset can cause the systems to produce false-positive alerts.

Some vehicles combine BSD with rear cross traffic alert (RCTA) systems.  For the current report, components related to RCTA were not included in the cost and weight analysis.

Figure 1: Typical Blind Spot Detection Zones

## Selection of Vehicles

Ricardo worked with NHTSA to define the study scope, baseline equipment, study assumptions, and cost methodology. The vehicle identification methodology is summarized in Figure 2.  The process involved three key procedures: 1) vehicle research, 2) vehicle segregation, and 3) vehicle selection.

Figure 2: Vehicle Identification/Selection Methodology

Specific vehicle selection criteria associated with the BSD systems included:

- Availability of information on vehicle BSD system

- Annual sales volumes

- Mix of domestic and foreign manufacturers

- Focus on systems developed by major suppliers of BSD technology

- To include variety of audible, visual and/or haptic display systems

- To be representative of high-volume, mainstream systems offered by key global suppliers

Six of the candidate vehicles meeting the stated criteria were evaluated in detail. These are summarized in Figure 3, which also indicates the three that were selected for disassembly, analysis, and cost & weight determinations.  A comparison of the three selected vehicles to the selection criteria is shown in Figure 4.  The Kia Sedona, Ford F-150 and Chrysler Pacifica were recommended for teardown analysis (includes cost and weight determination for each BSD system component).  These three vehicles offer the basic BSD features representative of typical mainstream vehicles.  The only warning method not represented is haptic.  It is noted the BMW & Audi systems shown in Figure 3 were not selected due to their associated higher-than-average costs.

Figure 3: Six candidate vehicles were studied in detail and three were selected for cost & weight analysis

Figure 4: Comparison of subject vehicles features to selection criteria

## BSD System Descriptions for Selected Vehicles

Kia Sedona:  Kia’s BSD System also supports Lane Change Assist (LCA) and Rear Cross Traffic Alert (RCTA) functions.

Figure 5: Location of Kia Sedona BSD Sensors (photo credit: Kia Sedona Owner’s Manual)

The Kia BSD is active when the selection switch is in the on position and vehicle speed is above 18.6 mph as shown in the upper left portion of Figure 6 (labeled 1).  The system will NOT alert the driver if the relative vehicle speed is 6 mph relative or greater compare to the other vehicle.

Alerts /Modes: BSD alerts the driver of a vehicle in the blind spot area with a 1st stage visual alert as shown in the middle right illustration in Figure 6.  The BSD /LCA system can be turned off with a lighted switch as shown in the upper right figure; the lighted switch was included in the weight and cost analysis.

Other Features: the LCA system causes the light to blink and an alarm to sound when a vehicle approaches at high speed as shown in illustration 2 of Figure 6 and if the turn signal is activated. This is the 2nd stage visual alarm as shown in the bottom left illustration.  Since the BSD system interfaces with the existing audible alarm system, none of the audible warning components were included in the cost and weight analysis.

Rear Cross Traffic Alert: RCTA is activated when the transmission is in reverse (R) mode and covers the area shown in illustration 3 of Figure 6.  The ultrasonic sensor(s) and cross traffic alert system components were not included in the BSD cost and weight analysis.

Figure 6: Kia Sedona BSD System Functions (photo credit: Kia Sedona Owner’s Manual)

Ford F-150:  Ford’s BSD system, known as the Blind Spot Information System (BLIS™), is integrated with a cross traffic alert.  The Continental 24 GHz radar sensors are located behind the tail light lenses on each side of the vehicle as illustrated in Figure 7.

The Ford system is activated when vehicle speed exceeds 2 mph and remains on when the transmission selector is in drive (D) or neutral (N) modes. The detection zone is on both sides of the vehicle from the location of the outside rear view mirror (ORVM) to approximately 10 feet beyond rear bumper.  The system is not activated for the condition of a vehicle passing rapidly through (< 2 seconds) the detection zone

Alerts/Modes: BLIS™ illuminates a light on the ORVM located on the same side of the vehicle from which the other vehicles is approaching.  The driver has the option of deactivating BLIS™ using a dash-mounted switch.

Other Features: The BLIS™ system is coupled with a Cross Traffic Alert when the transmission selector is not in drive (D) or neutral (N).  Hardware associated with the cross traffic alert system ultrasonic sensor(s) and other driver assist systems were not included in the cost and weight analysis.  Ford offers an optional, passive, integrated blind spot mirror on the driver’s side ORVM.  This was also not analyzed as it is not included as part of Ford’s BLIS system.

Figure 7: Ford F-150 BSD Radar and Visual Warning Locations

Chrysler Pacifica:  The Chrysler Pacifica includes user-selectable visual and audible blind spot monitoring (BSM) alerts.  The system includes 24 GHz radar sensors supplied by Autoliv and is operational in any forward transmission gear or reverse and monitors blind spots when the vehicle speed is greater than 6 miles/hour (see Figure 8).  The BSM is in standby mode when the transmission is in park.  The detection zone covers one lane width (12 ft) on both sides of the vehicle from the ORVM location to ~10 ft beyond the rear bumper.  The Pacifica owner’s manual states: “The Blind Spot Monitoring (BSM) system uses two radar based sensors, located inside the rear bumper fascia, to detect highway licensable vehicles (automobiles, trucks, motorcycles, etc.) that enter the blind spot zones from the rear/front/side of the vehicle.” The BSM system does not alert the driver about rapidly approaching vehicles that are outside the detection zones.

Alerts/Modes: The Chrysler BSM system detects degraded performance of the radar sensors due to contamination or foreign objects and displays a “blocked sensor” warning message and activates warning indicators in the outside rear view mirrors.  The warning indicators remain illuminated until blockage clearing conditions are met. The BSM system notifies the driver of objects in the detection zones by illuminating the BSM warning light located in the outside mirrors in addition to sounding an audible (chime) alert and reducing the radio volume.

- Blind spot alert lights only (default) – visual in ORVM

- Blind spot alert lights / chime – turn signal activates chime

- Blind spot alert off – No visual or audible alerts from either BSM or RCP systems

Other Features: Chrysler’s Rear Cross Path (RCP) system is activated when the transmission is in reverse.  The RCP includes ultrasonic sensor(s) which were not included in the cost and weight analysis.

Figure 8: Chrysler Pacifica Blind Spot Management System (photo credit: Chrysler Pacifica Owner’s Manual)

## Component Procurement and Disassembly

Key tasks for the cost and weight analysis include the procurement of parts associated with the BSD systems for the three selected vehicles, disassembly of these parts/components, identification of materials and processes used to produce the components, and measurement of the weights of the subassemblies, components, and parts. Typical BSD components are illustrated in Figure 9 and include:

- Sensors (typically radar, located near tail light assembly or integrated with this assembly)

- Control units (PCBs for sensors and antenna, LED control)

- Associated wiring harnesses

- Mounting hardware (specific to BSD system)

- Other materials and scrap (for electronic parts, this category includes  labels, soldering materials, flux, and fasteners)

Owner’s manuals, service manuals, OEM parts designations, and interviews with service technicians were employed to identify subassemblies, components and parts associated with the BSD system.  Detailed evaluations, including identification of sensors, displays, PCB pin assemblies, associated wire systems, and mounting hardware, ensured that only those items related to the BSD system were included in the cost and weight determinations.  The identified parts were then purchased through local dealers.

Figure 9: Typical BSD system components

Figure 10: Chrysler Pacifica Radar Sensor Locations and Associated Support Brackets (photo credit A2Mac1)

## Determination of Applicable Components

Many of the BSD components are integrated into or shared with other systems.  For example, connecting wires for the BSD system may be included in wire harnesses that interface with other, non-related equipment.  For the Kia Sedona, it was found that the BSD system utilizes a portion of the floor wiring harness for connecting and communicating with it subcomponents (see Figure 11).

Wires associated with systems other than BSD were not included in the cost and weight determinations.  Power connections, for example, exist independent of the BSD system and are thus not part of the incremental cost.  Based on the wire diagram of Figure 11, it was determined the portion of the floor harness associated with the BSD system included:

- 4 CAN wires connected between the 2 radars		wire length = 219 cm each

- 1 wire from left radar to BSD on/off switch		wire length = 534 cm

- 1 wire from left radar to left rear view mirror LED	wire length = 556 cm

- 1 wire from right radar to BSD on/off switch		wire length = 679 cm

- 1 wire from right radar to right rear view mirror LED	wire length = 556 cm

Figure 11: Portion of Kia Sedona Floor Wire Harness Associated with BSD System (photo credit: Kia Sedona Service Manual)

## Cost and Weight Analysis Approach

Ricardo engaged automotive system and vehicle integration experts, cost modeling teams, and procurement professionals to support the cost and weight analysis.  Following acquisition of the parts for dealers, analyses were performed to determine which subassemblies, components, and parts were uniquely associated with the BSD system.  The parts were then disassembled (“tear-down” procedure), catalogued, evaluated to determine manufacturing process, evaluated to determine materials used, weighed, and analyzed using specialized manufacturing cost software.

Figure 12: Weighing Kia Sedona Radar PCB (photo credit: MeC S.r.l.)

The analysis software facilitates organization of the disassembly process including a precise, systematic cataloguing system. Notes and measurements are included with the annotated photos.

Computational, experience-base, and heuristic procedures are followed in order to define the process, process times, and materials costs.  The results are summarized on standard cost and weight data forms.

## Costing Approach and Assumptions

The blind spot detection system components were torn down for cost and weight analysis of each individual elemental part and assembly costs were developed for each manufacturing step. Components that went through a series of production operations had the costs associated with those operations captured in the component costs. Standard parts such as fasteners were accounted for as procured parts in the analysis. Detailed manufacturing process operation worksheets are provided in the appendix for each of the analyzed components that illustrate how variable manufacturing costs, fixed burden, and weights are accumulated. These are then reconciled, each part to its subassembly, and from subassembly to the total incremental system.

An Asset Center Costing (ACC) methodology was used to identify cost drivers in terms of:

- Direct labor minutes per cycle

- Direct material costs per cycle

- Machine occupancy or station times per cycle

- Machinery, equipment, and tooling utilized and allocated per cycle.

The total manufacturing cost was built up from the following elements:

- Direct labor cost per unit (based on US rates for appropriate trades by manufacturing process)

- Direct material costs including scrap allowance and inbound freight per unit

- Variable burden/overhead costs, including indirect labor, energy, and other costs that vary with production volume

  - Indirect plant staff

  - Material handlers

  - 1st line supervision

  - Tool & equipment maintenance

  - Facilities maintenance

  - Non-production plant supplies

  - Energy

  - Process fluids & gases

  - Fringe on Direct labor

- Fixed burden / overhead per unit, including capital depreciation and other fixed costs

  - Capital equipment requiring investment and amortized at the stated annual sales volumes including property, plant machinery, equipment and primary tooling

    - Capital depreciation schedules for property, plant, and equipment

    - Special tooling depreciation schedules

  - Floor space for manufacturing and offices

  - Taxes – local and property

  - Insurance – property and liability

The following assumptions were utilized in applying the ACC methodology to the systems and components analyzed:

- Annual vehicle production volume of 200,000 units

- Burdened labor rate: labor rates were determined for specific manufacturing processes and applied to the cost analyses.  For example, the labor rate associated with injection molding operations located in the Midwest was assumed to be $33.66/hour.

- Capital equipment depreciation schedule of 12 years straight line with no residual value consistent with IRS depreciation schedules

- Special tooling depreciation schedule of 5 years straight line with no residual value consistent with IRS depreciation schedules

- Scrap rate of 1% of direct material cost based on an average for the automotive industry

In addition, overhead burdens for an automotive supplier and an automotive manufacturer were applied to get a more complete estimate of the contribution margin. The following corporate overhead rates were used as being typical for in-house made components by an original equipment manufacturer (OEM) in the light-duty automotive industry after reviewing Spinney et al, Rogozhin et al, National Academy of Sciences (NAS), and Vyas et al:

- SG&A of 8% applied to total manufacturing costs, including

  - Sales

  - Research & Development

  - General administration

  - Human resources

  - Supplier quality

  - Senior plant management

- Profit of 5% applied to total manufacturing cost

- Transportation and warranty costs of 10% applied to total manufacturing cost.

For outsourced components made by a Tier 1 supplier, however, the NAS found that outsourced components had a markup factor of only 75% that for in-house components as their Table 3.5 shows.

Therefore, the following corporate overhead rates for outsourced components, at 75% of the OEM in-house component rates, were used:

- SG&A of 6% applied to total manufacturing costs

- Profit of 3.75% applied to total manufacturing cost

- Transportation and warranty costs of 7.5% applied to total manufacturing cost.

These overhead burdens, along with the fixed burden and the variable manufacturing cost equate to a wholesale price from the manufacturer. Dealer costs and markup were estimated to be 11% of the wholesale price, consistent with Spinney et al, to arrive at a final end-user cost.

# 4.0 COST AND WEIGHT ANALYSIS

For each of the three selected vehicles, components related specifically to the BSD systems were identified as described in the Determination of Applicable Components section of 3.0 Engineering Analysis. The identified subassemblies were disassembled to individual components and analyzed for cost and weight. The parts were photographed, weighed and costed as per the approach outlined in the Costing Approach and Assumptions section of 3.0 Engineering Analysis.

## Cost and Weight Results

The following pages describe the vehicle components associated with the BSD systems and the results of the cost and weight analysis.  As noted above, once the components associated with the BSD systems were identified for the three selected vehicles, they were ordered through local dealers.

## Kia Sedona

The key components for the Kia Sedona BSD system are shown in the Table 1, which includes photos and descriptions of key parts acquired for the disassembly process.   It is noted that the “Mirror Assembly” components are those related to the BSD system and include the LED warning LED and associated hardware but not the other mirror components.

Table 1: Key Kia Sedona BSD Parts and Descriptions (photo credit MeC S.r.l.)

The complete results of the cost and weight analysis for the Kia Sedona are included in the appendix.  Table 2 provides a summary for the key subsystems as well as the full BSD system.

Table 2: Costs and weight summary for Kia Sedona BSD System and Key Components

The additional manufacturing cost of the Kia Sedona BSD system is estimated to be $129.27 with the radar assemblies accounting for 88%.  The total manufacturing costs are summarized by cost element as shown in Figure 13. The additional weight due to addition the BSD system is 1.35 kg.

Figure 13: Kia Sedona BSD System Cost Build-Up

The manufacturing cost increase of the BSD system is shown in Table 3.

Table 3:  Sedona Kia BSD System Manufacturing Cost Increase

## Ford F-150

The key components for the Ford F-150 BSD system are shown in Table 4 which includes photos and descriptions of key parts acquired for the disassembly process.  The F-150 BSD system includes two identical radar sensor assemblies and the mirror-mounted LED warning light.

The complete results of the cost and weight analysis for the Ford F-150 are included in the appendix.  Table 5 provides a summary for the key subsystems as well as the full BSD system.

Table 4: Key Ford F-150 BSD System Parts and Descriptions (photo credit MeC S.r.l.)

Table 5: Costs and weight summary for Ford F-150 BSD System and Key Components

The additional manufacturing cost of the Ford F-150 BSD system is estimated to be $92.70 with the radar assemblies accounting for 64%.  These costs are summarized in Figure 14. The additional weight due to addition the BSD system is 1.28 kg.

Figure 14: Ford F-150 BSD System Cost Build-Up

The manufacturing cost increase of the BSD system is shown in Table 6.

Table 6:  Ford F-150 BSD System Manufacturing Cost Increase

## Chrysler Pacifica

The key components for the Chrysler Pacifica BSD system are shown in Table 7 which includes photos and descriptions of key parts acquired for the disassembly process.  It is noted that the Pacifica includes two identical radar sensors and ORVM visual warning displays incorporating an LED and associated PCB driver and wiring.

The complete results of the cost and weight analysis for the Chrysler Pacifica are included in the appendix.  Table 8 provides a summary for the key subsystems as well as the full BSD system.

Table 7: Key Chrysler Pacifica BSD System Parts and Descriptions (photo credit MeC S.r.l.)

Table 8: Costs and weight summary for Chrysler Pacifica BSD System and Key Components

The additional manufacturing cost of the Chrysler Pacifica BSD system is estimated to be $114.41 with the radar assemblies accounting for 83%.  These costs are summarized in Figure 15. The additional weight due to addition the BSD system is 0.43 kg.

Figure 15: Chrysler Pacifica BSD System Cost Build-Up

The manufacturing cost increase of the BSD system is shown in Table 9.

Table 9:  Chrysler Pacifica BSD System Manufacturing Cost Increase

## Comparison of Evaluated BSD Systems and Summary

Table 10 shows comparisons of the study results for the three BSD Systems.  Costs to the manufacturer range from $92.70 to $129.27, with an average of $112.13.  The distribution of cost components for the average system is shown in the pie chart of Figure 17.

The added weight of the three BSD systems is summarized in Table 10 and Figure 18.  They range from 0.43 kg to 1.35 kg, with an average of 0.89 kg.

The incremental prices of the three BSD systems to the end user are summarized in Table 11.

Figure 16: Comparisons of BSD Systems Manufacturing Costs

Figure 17: Manufacturing Cost Components for Average BSD System.

Table 10: Costs and weight summary for three BSD Systems

The weight of the Chrysler Pacifica BSD system is significantly lower than that for the other two vehicles.  This is because the support brackets and housing are fabricated from lightweight plastic compared to aluminum and steel for the other two vehicles.

Figure 18: Incremental Weight of Three BSD Systems (use Ricardo color scheme)

Table 11: Average Incremental Price of BSD Systems to End User

# 5.0 ACRONYMS

ADAS		Advanced Driver Assist Systems

AEB		Advanced Emergency Braking

BOM		Bill of Materials

BSD 		Blind Spot Detection

BLIS™		Ford Blind Spot Information System

CIB		Crash Imminent Braking

CO 		NHTSA Contracting Officer

COR(TO) 	NHTSA Contracting Officer’s Representative - Task Order

DBS		Dynamic Brake Support

DCM		Door Control Module

FCW		Forward Collision Warning

LCA		Lane Change Assist

LDWS		Lane Departure Warning System

ORVM		Outside Rear View Mirror

PCB		Printed Circuit Board

NHTSA 	National Highway Traffic Safety Administration

RCP		Rear Cross Path

RCTA		Rear Cross Traffic Alert

RSC		Ricardo Strategic Consulting

V2V		Vehicle-to-Vehicle Communication

# 6.0 APPENDICES

The appendices include: 1) bills of material (BOM) for each of the three selected vehicles, 2) cost and weight summary tables for each of the three selected vehicles.

## BOM for Kia Sedona

The Kia Sedona BSD system includes two radar system PCBs.  Based on their color, these two PCBs are referred to as the “Green Radar PCB” and “White Radar PCB”.  A photo of the top and bottom sides of these two PCBs is shown below, followed by the corresponding BOMs.

Photo of top and bottom sides of Kia Sedona Radar PCBs (photo credit MeC S.r.l.)

## Cost and Weight Summary for Kia Sedona

Details of the BSD cost and weight analysis of the Kia Sedona is shown in the table below:

## BOM for Ford F-150

Similar to the Kia Sedona, the Ford F-150 includes two radar PCBs, labeled “Green” and “White” (antenna).  These are shown in the photos below, followed by the corresponding BOMs.

Photo of top and bottom sides of Ford F-150 Radar PCBs (photo credit MeC S.r.l.)

## Cost and Weight Summary for Ford F-150

## BOM for Chrysler Pacifica

Similar to the Kia Sedona and the Ford F-150, the Chrysler Pacifica includes two radar PCBs, labeled “Green” and “White” (antenna).  These are shown in the photos below, followed by the corresponding BOMs.

Chrysler Pacifica BSD System Radar PCBs (photo credit MeC S.r.l.)

## Cost and Weight Summary for Chrysler Pacifica

Demandt, B., et.al., CarSalesBase, U.S. Car Sales Data, http://carsalesbase.com/us-car-sales-data/

Forkenbrock, G., Hoover, R. L., Gerdus, E., Van Buskirk, T. R., & Heitz, M. (2014, July). Blind spot monitoring in light vehicles — System performance. (Report No. DOT HS 812 045). Washington, DC: National Highway Traffic Safety Administration.

Ibid, Page 2.

Cars Sales Base, http://carsalesbase.com/us-car-sales-data/

Bennink, C., Pickup Truck Gear: New Technology Puts Safety into the Pickup Truck Buying Process, For Construction Pros, 15 May 2015, https://www.forconstructionpros.com/trucks/trucks-accessories/article/12067915/pickup-truck-gear-new-technology-puts-safety-into-the-pickup-buying-process

Spinney, B.C., Faigin, B., Bowie, N., & St. Kratzke. 1999, Advanced Air Bag Systems Cost, Weight, and Lead Time analysis Summary Report, Contract NO. DTNH22-96-0-12003, Task Orders – 001, 003, and 005. Washington, D.C., U.S. Department of Transportation

Rogozhin, A., Gallaher, M., & McManus, W. 2009, Automobile Industry Retail Price Equivalent and Indirect Cost Multipliers.  Report by RTI International to Office of Transportation Air Quality.  U.S. Environmental Protection Agency, RTI Project Number 0211577.002.004, February, Research Triangle Park, N.C.

National Academy of Sciences, Committee on the Assessment of Technologies for Improving Light-Duty Vehicle Fuel Economy, "Assessment of Fuel Economy Technologies for Light-Duty Vehicles," The National Academies Press, Washington D.C., 2011

Vyas, A., Santini, D., And Cuenca, R., "Comparison of Indirect Cost Multipliers for Vehicle Manufacturing," Technical Memorandum of the Center for Transportation Research, Argonne National Laboratory, April 2000

See p. 33 of National Academy of Sciences, Committee on the Assessment of Technologies for Improving Light-Duty Vehicle Fuel Economy, "Assessment of Fuel Economy Technologies for Light-Duty Vehicles," The National Academies Press, Washington D.C., 2011